HILLSBOROUGH COLLEGE

AI Innovation Center

K1 Humanoid Robot

AI Voice Pipeline Setup Manual

K1 Mic Array  →  Whisper STT  →  Llama 3  →  Piper TTS  →  K1 Speaker + Action

WHAT'S NEW: Version 2.0 — Updated to use the K1's built-in microphone array and speaker via the official Booster Audio SDK. No external microphone required. LLM API options available.

AI PREP4WORK Initiative — FIPSE Grant

Dale Mabry Campus — Tampa, Florida

Version 2.0 — 2026

# Overview

This manual guides you through building a fully local AI voice pipeline for the Booster K1 humanoid robot. The K1 listens through its own built-in microphone array, thinks using Llama 3 running on your laptop, speaks through its own speaker, and moves — all without any cloud services or external microphone.

This pipeline uses the official Booster Audio SDK to access the K1's hardware audio directly, replacing the default Doubao LLM with a fully customizable open-source stack.

| Stage | Tool | Runs On | What It Does |
| --- | --- | --- | --- |
| 1. Listen | AudioCaptureStream + Whisper | K1 mic → Laptop | Streams PCM audio from K1's mic array, transcribes to text |
| 2. Think | Llama 3 via Ollama | Laptop | Generates a spoken response and chooses a movement action |
| 3. Speak | Piper TTS + AudioPlayer | Laptop → K1 speaker | Converts text to WAV, plays through K1's speaker |
| 4. Move | Booster SDK (Python) | Laptop → K1 | Sends walk/gesture commands to the K1 over WiFi |
| 5. (Optional) Volume | AudioSystemController | Laptop → K1 | Adjusts K1 master volume from Python |

KEY ADVANTAGE: The K1's microphone array includes NAEC (Network Acoustic Echo Cancellation), which prevents the robot from hearing its own voice during playback. This is essential for a continuous conversation loop.

# Requirements

## Hardware

- Booster K1 EDU humanoid robot (powered on, connected to WiFi)

- Instructor laptop (Windows, macOS, or Linux) — no external microphone needed

- Both laptop and K1 on the same WiFi network

## Laptop Specs (Minimum)

- 16 GB RAM

- 8 GB free disk space (models + tools)

- Python 3.10 or higher

- Ollama already installed with Llama 3 pulled and working

PREREQUISITE: This manual assumes Ollama and Llama 3 are already installed and working on your laptop.

# Step 1: Find the K1's IP Address

Your laptop needs the K1's WiFi IP address to connect. The K1's default wired Ethernet IP is 192.168.10.102, but its WiFi IP is assigned by your router and will be different.

## Option A: Via the Booster Mobile App

- Power on the K1 and wait for full boot (~60 seconds).

- Open the Booster app on your phone and connect to the robot.

- Navigate to Settings — the WiFi IP address is displayed there.

## Option B: Via SSH Over Ethernet

- Connect your laptop to the K1 with an Ethernet cable.

- SSH into the robot:

ssh booster@192.168.10.102   # Password: 123456

- Run the following to find the WiFi IP:

ip addr show wlan0

- Look for the inet line (e.g., 192.168.1.105). That is your WiFi IP.

YOUR K1 IP: Write the K1's WiFi IP here: ___________________________ You will use this throughout the setup.

# Step 2: Install the Booster Robotics SDK

The Booster SDK provides Python access to the K1's movement controls and all audio hardware including the microphone array and speaker.

## 2a. Install Build Dependencies

On Ubuntu/Linux:

sudo apt-get update

sudo apt-get install -y cmake build-essential git

On macOS:

brew install cmake git

On Windows:

- Install Git: https://git-scm.com

- Install CMake: https://cmake.org/download

- Install Visual Studio Build Tools (select C++ workload)

## 2b. Clone and Build the SDK

git clone https://github.com/BoosterRobotics/booster_robotics_sdk.git

cd booster_robotics_sdk

mkdir build && cd build

cmake .. -DBUILD_PYTHON_BINDING=ON

make -j$(nproc)

sudo make install

## 2c. Verify the Installation

python3 -c "import booster_robotics_sdk_python; print('SDK OK')"

You should see: SDK OK

TROUBLESHOOTING: If the import fails, run: export PYTHONPATH=/usr/local/lib/python3.x/site-packages:$PYTHONPATH replacing 3.x with your Python version.

# Step 3: Install Whisper (Speech-to-Text)

Whisper converts the audio streamed from the K1's microphone array into text. It runs entirely on your laptop.

## 3a. Install Whisper and Audio Libraries

pip install openai-whisper

pip install numpy scipy soundfile

## 3b. Download the Whisper Model (One Time)

python3 -c "import whisper; whisper.load_model('medium')"

| Model | Speed | Best For |
| --- | --- | --- |
| tiny | Very fast | Quick demos, controlled environments |
| base | Fast | General classroom use |
| medium | Moderate | Recommended — best balance of speed and accuracy |
| large | Slow | Best accuracy, requires more RAM |

# Step 4: Install Piper TTS (Text-to-Speech)

Piper converts Llama 3's text response into a WAV file that is then played through the K1's speaker using the Audio SDK. Piper is the same TTS engine used on the AI Innovation Center's PiDog (Diggy).

## 4a. Install Piper

pip install piper-tts

## 4b. Download a Voice Model

mkdir -p ~/piper_voices && cd ~/piper_voices

wget https://huggingface.co/rhasspy/piper-voices/resolve/main/en/en_US/lessac/medium/en_US-lessac-medium.onnx

wget https://huggingface.co/rhasspy/piper-voices/resolve/main/en/en_US/lessac/medium/en_US-lessac-medium.onnx.json

## 4c. Test Piper

echo "Hello from K1." | piper \

--model ~/piper_voices/en_US-lessac-medium.onnx \

--output_file /tmp/test.wav

aplay /tmp/test.wav   # Linux  |  afplay /tmp/test.wav  (macOS)

# Step 5: Prepare the K1 for Remote Control

## 5a. Power On Sequence

- Place the K1 on its stand in a clear 3m × 3m area with a spotter present.

- Power on and wait for full boot (~60 seconds).

- Confirm the K1 is connected to the same WiFi network as your laptop.

## 5b. Start the Robot Control Service

ssh booster@<K1-IP-ADDRESS>   # Password: 123456

booster-cli launch -c start

## 5c. Safe Mode Sequence (REQUIRED)

SAFETY CRITICAL: Always follow DAMP → PREP → WALK order. Never skip PREP. The Python script handles this automatically — do not interrupt it during startup.

- DAMP — robot is limp, motors off (starting state)

- PREP — robot stands and stiffens joints (hold for 3 seconds minimum)

- WALK — robot is ready for movement commands

# Step 6: Understanding the K1 Audio SDK

The Booster Audio SDK gives Python direct access to the K1's built-in audio hardware. This section explains the components used in the pipeline so you can customize it later.

| SDK Class | What It Does in This Pipeline |
| --- | --- |
| AudioManager | Entry point — must be initialized first; creates all other audio objects |
| AudioCaptureStream | Streams raw PCM audio frames from the K1's microphone array to your laptop in real time |
| AudioPlayer | Plays a WAV file through the K1's built-in speaker |
| AudioSystemController | Adjusts the K1's master volume from Python (optional) |
| AudioRecorder | Records mic audio to a file on the robot (not used in main pipeline, useful for debugging) |
| AudioLocalizer | Returns the direction a sound is coming from — future use for tracking speakers |

## Audio Flow Explained

- AudioCaptureStream uses NAEC (echo cancellation) so the K1 will not hear its own speaker output. Always enable NAEC in the capture stream.

- AudioPlayer accepts a WAV file path. Piper generates this file on your laptop, then it is transferred to the robot and played.

- The AudioManager must be initialized before any other audio object is created, and Shutdown() must be called before the program exits.

- Always register callbacks before calling Init() on any audio object so you do not miss the first state change.

# Step 7: The Main Pipeline Script

Save the following as k1_voice_pipeline.py on your laptop. Replace 192.168.1.XXX with your K1's actual IP address on line 10.

WHAT THIS DOES: This script streams audio from the K1's mic, transcribes with Whisper, generates a response and action with Llama 3, synthesizes speech with Piper, copies the WAV to the robot, and plays it through the K1's speaker — then moves the robot.

import whisper

import numpy as np

import requests, subprocess, time, os, tempfile, queue

import soundfile as sf

from booster_robotics_sdk_python import (

ChannelFactory, RobotClient, RobotMode,

AudioManager, AudioCaptureStream, AudioPlayer,

AudioSystemController, PlayerInitOptions,

AudioSourceType, CaptureStreamInitOptions,

PlayerPriority

)

# ── CONFIGURATION ─────────────────────────────────────────────────

K1_IP           = "192.168.1.XXX"   # <-- Replace with your K1's IP

OLLAMA_URL      = "http://localhost:11434/api/generate"

WHISPER_MODEL   = "medium"

PIPER_MODEL     = os.path.expanduser(

"~/piper_voices/en_US-lessac-medium.onnx")

K1_AUDIO_TMP    = "/tmp/k1_response.wav"  # path on the ROBOT

LOCAL_WAV_TMP   = "/tmp/response.wav"      # path on YOUR LAPTOP

SAMPLE_RATE     = 16000

CAPTURE_SECONDS = 5

# ── SYSTEM PROMPT ─────────────────────────────────────────────────

SYSTEM_PROMPT = """

You are K1, a friendly humanoid robot at Hillsborough College.

Keep responses to 1-2 sentences.

End every response on a NEW LINE with ONE action tag:

[ACTION:walk_forward]

[ACTION:wave]

[ACTION:nod]

[ACTION:none]

"""

# ── INITIALIZE COMPONENTS ─────────────────────────────────────────

print("Loading Whisper...")

asr = whisper.load_model(WHISPER_MODEL)

print("Connecting to K1...")

ChannelFactory.Instance().Init(0, K1_IP)

robot = RobotClient()

robot.Init()

time.sleep(2)

print("Initializing K1 audio...")

audio_mgr = AudioManager()

audio_mgr.Init()

# Set volume to 80%

vol_ctrl = audio_mgr.CreateSystemController()

vol_ctrl.SetVolume(80)

# ── AUDIO CAPTURE (K1 MIC ARRAY) ──────────────────────────────────

audio_queue = queue.Queue()

def on_audio_frame(frame):

# NAEC-processed PCM frames arrive here in real time

audio_queue.put(bytes(frame.naec_pcm_data))

capture = audio_mgr.CreateCaptureStream()

capture.SetFrameCallback(on_audio_frame)

cap_opts = CaptureStreamInitOptions()

cap_opts.enable_raw_pcm  = False   # Raw mic (picks up speaker too)

cap_opts.enable_naec_pcm = True    # Echo-cancelled mic (recommended)

cap_opts.sample_rate     = SAMPLE_RATE

cap_opts.channels        = 1

cap_opts.bits_per_sample = 16

capture.Init(cap_opts)

# ── HELPER FUNCTIONS ──────────────────────────────────────────────

def record_from_k1():

"""Collect CAPTURE_SECONDS of NAEC audio from K1 mic array."""

print("Listening via K1 mic...")

frames = []

capture.Start()

deadline = time.time() + CAPTURE_SECONDS

while time.time() < deadline:

try:

chunk = audio_queue.get(timeout=0.1)

frames.append(np.frombuffer(chunk, dtype=np.int16))

except queue.Empty:

pass

capture.Stop()

audio = np.concatenate(frames).astype(np.float32) / 32768.0

sf.write(LOCAL_WAV_TMP, audio, SAMPLE_RATE)

return LOCAL_WAV_TMP

def transcribe(wav_path):

result = asr.transcribe(wav_path)

text = result['text'].strip()

print(f"Heard: {text}")

return text

def ask_llama(user_text):

payload = {

"model": "llama3",

"prompt": f"{SYSTEM_PROMPT}\nUser: {user_text}\nK1:",

"stream": False

}

r = requests.post(OLLAMA_URL, json=payload)

return r.json()['response']

def speak_on_k1(response_text):

"""Generate WAV with Piper, copy to robot, play via AudioPlayer."""

spoken = response_text.split('[ACTION:')[0].strip()

# Generate WAV on laptop

subprocess.run([

'piper', '--model', PIPER_MODEL,

'--output_file', LOCAL_WAV_TMP

], input=spoken.encode(), check=True)

# Copy WAV to robot over SSH

subprocess.run([

'scp', '-o', 'StrictHostKeyChecking=no',

LOCAL_WAV_TMP,

f'booster@{K1_IP}:{K1_AUDIO_TMP}'

], check=True)

# Play on K1's speaker via Audio SDK

player = audio_mgr.CreatePlayer()

player.SetStateCallback(lambda s: None)  # optional

opts = PlayerInitOptions()

opts.source_type = AudioSourceType.kWavFile

opts.source_uri  = K1_AUDIO_TMP

opts.priority    = PlayerPriority.kMedium

player.Init(opts)

player.Start()

time.sleep(3)   # wait for playback

player.Destroy()

def execute_action(response_text):

if '[ACTION:walk_forward]' in response_text:

print("Action: walking forward")

robot.ChangeMode(RobotMode.kWalk)

time.sleep(3)

robot.Move(0.3, 0, 0)   # x=forward, y=lateral, z=rotation

time.sleep(2)

robot.Move(0, 0, 0)     # stop

elif '[ACTION:wave]' in response_text:

print("Action: waving")

# Add custom joint commands here using the Booster SDK

elif '[ACTION:nod]' in response_text:

print("Action: nodding")

# Add head pitch joint commands here

else:

print("Action: none")

# ── STARTUP SEQUENCE ──────────────────────────────────────────────

print("Setting robot to PREP mode...")

robot.ChangeMode(RobotMode.kPrepare)

time.sleep(3)   # REQUIRED: hold PREP for 3 seconds minimum

print("K1 pipeline ready. Ctrl+C to stop.")

# ── MAIN LOOP ─────────────────────────────────────────────────────

while True:

try:

wav_path  = record_from_k1()

user_text = transcribe(wav_path)

if not user_text:

continue

response = ask_llama(user_text)

print(f"K1 says: {response}")

speak_on_k1(response)

execute_action(response)

time.sleep(1)

except KeyboardInterrupt:

print("Shutting down safely...")

capture.Destroy()

audio_mgr.Shutdown()

robot.ChangeMode(RobotMode.kDamp)

break

except Exception as e:

print(f"Error: {e} — continuing...")

time.sleep(2)

# Step 8: Run the Pipeline

## Start Ollama First

# Terminal 1

ollama serve

# Verify Llama 3 is available

ollama list

If llama3 is not listed, run: ollama pull llama3

## Run the Pipeline

# Terminal 2

python3 k1_voice_pipeline.py

Expected startup sequence:

- Loading Whisper... (first run: ~30 seconds)

- Connecting to K1...

- Initializing K1 audio...

- Setting robot to PREP mode...

- K1 pipeline ready. Ctrl+C to stop.

- Listening via K1 mic... (speak now)

USAGE TIP: Speak clearly toward the K1. Pause briefly after your question. The robot will respond, speak through its own speaker, and move based on the conversation.

# Customization Guide

## Changing the Robot's Personality

Edit the SYSTEM_PROMPT variable. Example for a student-facing tutor:

SYSTEM_PROMPT = """

You are K1, a robotics tutor at Hillsborough College.

Help students understand AI and robotics concepts.

Be encouraging. Keep answers to 2 sentences.

End with: [ACTION:walk_forward], [ACTION:wave],

[ACTION:nod], or [ACTION:none]

"""

## Adding New Actions

elif '[ACTION:turn_right]' in response_text:

robot.Move(0, 0, 0.5)   # rotation

time.sleep(2)

robot.Move(0, 0, 0)

## Adjusting Listen Duration

Change CAPTURE_SECONDS at the top of the script. 5 seconds works well for short commands. Use 8-10 for longer questions.

## Adjusting Volume

Change the SetVolume() call. Value is 0-100.

vol_ctrl.SetVolume(60)   # quieter

vol_ctrl.SetVolume(100)  # maximum

# Replication at Other Colleges

This pipeline is designed to be portable. Any college can replicate it with the following minimum requirements:

- Booster K1 EDU robot

- Laptop: Windows, macOS, or Linux with at least 16 GB RAM

- 8 GB free disk space (Whisper + Llama 3 + Piper models)

- Local WiFi network (no internet required after initial setup)

- No external microphone needed — K1's built-in mic array is used

| Setup Task | Estimated Time |
| --- | --- |
| Install Ollama + pull Llama 3 | 15-30 min (download varies by connection) |
| Build and install Booster SDK | 30-60 min |
| Install Whisper + download model | 10 min install + ~20 min download |
| Install Piper + download voice | 10 min install + ~5 min download |
| Configure and test pipeline | 30 min |
| Total (first time) | ~2-3 hours |

# Appendix A: Using Cloud API Keys Instead of Ollama

The default pipeline uses Llama 3 running locally via Ollama — no internet required and no ongoing cost. However, if your institution has an OpenAI or Anthropic API key, you can swap the LLM step to use a cloud model instead. Everything else in the pipeline (K1 mic, Whisper, Piper, movement) stays exactly the same.

| Option | Model | Cost | Requires Internet | Best For |
| --- | --- | --- | --- | --- |
| Ollama (default) | Llama 3 (local) | Free | No | Offline labs, no budget |
| OpenAI API | GPT-4o or GPT-4o-mini | Pay per use | Yes | Best cloud accuracy |
| Anthropic API | Claude Sonnet or Haiku | Pay per use | Yes | Strong reasoning, safety |

KEY POINT: You only need to change the ask_llm() function. All other steps — audio capture, Whisper, Piper, movement — are identical regardless of which LLM you choose.

## Option A: OpenAI API

Install and Configure

pip install openai

Set your API key. On Linux/macOS — add to ~/.bashrc or ~/.zshrc:

export OPENAI_API_KEY="sk-...your-key-here..."

On Windows — System Properties → Environment Variables:

Variable: OPENAI_API_KEY

Value:    sk-...your-key-here...

Replace ask_llama() With This Function

In k1_voice_pipeline.py, delete the ask_llama() function and replace it with:

import os

from openai import OpenAI

openai_client = OpenAI(api_key=os.environ.get('OPENAI_API_KEY'))

def ask_llm(user_text):

response = openai_client.chat.completions.create(

model="gpt-4o-mini",   # or "gpt-4o" for best quality

messages=[

{"role": "system", "content": SYSTEM_PROMPT},

{"role": "user",   "content": user_text}

],

max_tokens=200

)

return response.choices[0].message.content.strip()

Also update the main loop: change ask_llama(user_text) to ask_llm(user_text).

| Model | Speed | Cost | Best For |
| --- | --- | --- | --- |
| gpt-4o-mini | Fast | ~$0.15/1M tokens | Everyday demos, student interactions |
| gpt-4o | Moderate | ~$5.00/1M tokens | Complex reasoning, research demos |

## Option B: Anthropic API (Claude)

Install and Configure

pip install anthropic

On Linux/macOS:

export ANTHROPIC_API_KEY="sk-ant-...your-key-here..."

On Windows — Environment Variables:

Variable: ANTHROPIC_API_KEY

Value:    sk-ant-...your-key-here...

Replace ask_llama() With This Function

import os

import anthropic

anthropic_client = anthropic.Anthropic(

api_key=os.environ.get('ANTHROPIC_API_KEY')

)

def ask_llm(user_text):

message = anthropic_client.messages.create(

model="claude-haiku-4-5-20251001",  # fast + affordable

max_tokens=200,

system=SYSTEM_PROMPT,

messages=[

{"role": "user", "content": user_text}

]

)

return message.content[0].text.strip()

| Model | Speed | Cost | Best For |
| --- | --- | --- | --- |
| claude-haiku-4-5-20251001 | Very fast | Low | Recommended — fast live demos |
| claude-sonnet-4-6 | Moderate | Mid | More nuanced responses, curriculum Q&A |

NOTE FOR INSTRUCTORS: Claude models follow Anthropic’s safety guidelines by default, which may be an advantage for student-facing deployments. Your SYSTEM_PROMPT still fully controls K1’s persona and behavior.

## Keeping API Keys Secure

API keys are like passwords. Follow these practices in a classroom or lab:

- Never paste a key directly into the script file. Always use environment variables.

- Do not share scripts with a key embedded — share only the template with a placeholder.

- Use a .env file with python-dotenv if you prefer not to set environment variables manually:

# .env file  (never commit this to GitHub)

OPENAI_API_KEY=sk-...your-key...

ANTHROPIC_API_KEY=sk-ant-...your-key...

pip install python-dotenv

# Add to top of k1_voice_pipeline.py

from dotenv import load_dotenv

load_dotenv()

- If sharing with students, create a read-only key with usage limits in your provider’s dashboard.

- Monitor usage on the OpenAI or Anthropic dashboard weekly during active labs.

RECOMMENDATION FOR PARTNER COLLEGES: For partner colleges replicating this setup: Ollama with Llama 3 is always the recommended default — zero ongoing cost, works offline. Cloud APIs are an optional upgrade when a budget is available.

# Troubleshooting

## No Audio from K1 Mic (Empty Frames)

- Confirm AudioManager.Init() completed without error

- Make sure NAEC is supported: the K1 EDU includes the circular microphone array

- Try enable_raw_pcm = True as a fallback to verify audio is flowing at all

- Check that the K1's audio service is running (it starts with booster-cli launch)

## K1 Speaker Silent

- Verify the WAV file was copied to the robot: SSH in and run ls -la /tmp/k1_response.wav

- Check volume: vol_ctrl.SetVolume(100) and try again

- Confirm source_uri in PlayerInitOptions is the ROBOT path (/tmp/...), not your laptop path

- Make sure player.Init() returns without error before calling Start()

## Robot Not Moving

- Verify the robot went through PREP mode and the 3-second wait completed

- Check that ChangeMode(RobotMode.kWalk) is called before Move()

- Confirm the K1 IP address is correct and the control service is running

## Whisper Transcription Errors

- Move to a quieter environment or increase CAPTURE_SECONDS

- Switch to the large Whisper model for better accuracy in noisy spaces

- Speak directly toward the K1's head where the mic array is located

## Ollama / Llama 3 Not Responding

- Run ollama serve in a separate terminal before starting the pipeline

- Verify: ollama list — if llama3 is missing, run: ollama pull llama3

# Quick Reference Card

Print this page and keep it at the robot station.

| STARTUP CHECKLIST |  |
| --- | --- |
| 1 | Power on K1, wait 60 seconds, confirm WiFi connected |
| 2 | SSH in: ssh booster@<K1-IP> then run: booster-cli launch -c start |
| 3 | On laptop — Terminal 1: ollama serve |
| 4 | On laptop — Terminal 2: python3 k1_voice_pipeline.py |
| 5 | Wait for 'K1 pipeline ready' |
| 6 | Speak toward the K1's head. Pause after each question. |
| 7 | Press Ctrl+C to shut down safely (robot returns to DAMP mode) |

SHUTDOWN: Ctrl+C in Terminal 2. Robot returns to DAMP (safe) mode automatically.

Hillsborough College — AI Innovation Center — AI PREP4WORK Initiative

This document may be shared freely with partner institutions.