HILLSBOROUGH COLLEGE

AI Innovation Center

K1 Humanoid Robot

Webots Simulator Setup Manual

Windows 11  →  WSL2  →  Docker  →  Webots  →  K1 Simulation

PURPOSE: This manual covers the complete path from a fresh Windows 11 laptop to a fully working Booster K1 robot simulator, so instructors can test code without the physical robot present.

AI PREP4WORK Initiative — FIPSE Grant

Dale Mabry Campus — Tampa, Florida

Version 1.0 — 2026

# Overview

This manual walks you through setting up a Booster K1 robot simulator on a Windows 11 laptop. Once complete, you can test all Python pipeline code — including movement commands, voice responses, and LLM integration — against a simulated K1 robot without needing the physical hardware present.

This is especially useful for:

- Testing new code before running it on the real K1

- Allowing students to develop and experiment without robot access

- Replicating the AI Innovation Center setup at partner colleges before hardware arrives

## How It Works

Windows 11 cannot run the Booster Webots simulator directly. The simulator requires a Linux environment. The solution is WSL2 — Windows Subsystem for Linux — which runs a real Ubuntu Linux environment inside Windows, with full access to your NVIDIA GPU.

| Layer | What It Is | Why It Is Needed |
| --- | --- | --- |
| Windows 11 | Your existing OS | Host system |
| WSL2 | Ubuntu Linux inside Windows | Required to run Linux-based simulator tools |
| NVIDIA Driver (Windows) | GPU driver on Windows side only | Passes GPU through to WSL2 automatically |
| Docker (inside WSL2) | Container runtime | Runs the pre-built Webots + K1 simulation environment |
| Webots K1 Container | Booster’s simulator package | 3D robot simulation with Booster SDK connection |
| Your Python script | k1_voice_pipeline.py | Connects to simulator the same way it connects to the real robot |

YOUR HARDWARE: Your NVIDIA RTX 1000 Ada fully supports GPU passthrough to WSL2. No separate Linux driver is needed inside WSL2 — the Windows driver handles everything.

# Requirements

| Requirement | Minimum | Your Laptop |
| --- | --- | --- |
| Operating System | Windows 11 | ✓ Windows 11 |
| GPU | NVIDIA with CUDA support | ✓ RTX 1000 Ada |
| RAM | 16 GB | Confirm in Task Manager |
| Disk Space | 40 GB free | Check in File Explorer |
| NVIDIA Driver | Version 530+ (Windows) | Check below |
| Virtualization | Enabled in BIOS | Check below |

## Pre-Check 1: Verify Your NVIDIA Driver Version

Open PowerShell (search Start menu) and run:

nvidia-smi

Look for Driver Version. It must be 530 or higher. If lower, download the latest Studio Driver from nvidia.com/drivers.

## Pre-Check 2: Verify Virtualization is Enabled

Press Ctrl + Shift + Esc to open Task Manager, click the Performance tab, then CPU. Look for Virtualization: Enabled. If it shows Disabled, you need to enable it in your laptop’s BIOS before proceeding.

IF VIRTUALIZATION IS DISABLED: On most laptops Virtualization is enabled by default. If it is disabled, restart the laptop, press F2 or Delete during startup to enter BIOS, find Virtualization Technology, and enable it.

# Part 1: Install WSL2 with Ubuntu

WSL2 (Windows Subsystem for Linux 2) gives you a full Ubuntu Linux environment running inside Windows 11. This is the foundation everything else builds on.

## Step 1a: Install WSL2

Open PowerShell as Administrator (right-click Start menu → Windows Terminal (Admin) or PowerShell (Admin)) and run:

wsl --install

This single command installs WSL2, the Linux kernel, and Ubuntu 24.04 automatically. Restart your laptop when prompted.

EXISTING WSL USERS: If you already have WSL1, upgrade it: wsl --set-default-version 2 then wsl --update

## Step 1b: First Launch and User Setup

- After restarting, search for Ubuntu in the Start menu and open it.

- Wait for the first-time setup to complete (~2 minutes).

- When prompted, enter a Linux username (lowercase, no spaces — example: hillsborough).

- Enter a password. This is your WSL2 sudo password — remember it.

- You should see a $ prompt. WSL2 is working.

## Step 1c: Configure WSL2 Memory Allocation

By default WSL2 only uses 50% of your RAM, which may not be enough for Webots. Set it higher:

Open Notepad on Windows and create the file C:\Users\YourUsername\.wslconfig with this content:

[wsl2]

memory=12GB

swap=8GB

processors=4

localhostForwarding=true

networkingMode=mirrored

[experimental]

autoMemoryReclaim=dropcache

sparseVhd=true

Replace YourUsername with your actual Windows username. Save the file, then restart WSL2 from PowerShell:

wsl --shutdown

Then reopen Ubuntu from the Start menu.

## Step 1d: Update Ubuntu

Inside the Ubuntu terminal, run:

sudo apt update && sudo apt upgrade -y

## Step 1e: Verify GPU Passthrough

Still inside Ubuntu, run:

nvidia-smi

You should see your RTX 1000 Ada listed with the driver version. If you see it, GPU passthrough is working perfectly.

CRITICAL WARNING: Do NOT install any NVIDIA driver inside WSL2. The Windows driver handles everything automatically. Installing a Linux NVIDIA driver inside WSL2 will break the GPU passthrough.

# Part 2: Install Docker with NVIDIA Support

Docker runs the pre-built Booster K1 Webots simulation container. You install Docker inside WSL2 Ubuntu, not on Windows directly.

## Step 2a: Install Docker Engine

Run these commands inside your Ubuntu WSL2 terminal:

sudo apt update

sudo apt install -y ca-certificates curl gnupg

# Add Docker's official GPG key

sudo install -m 0755 -d /etc/apt/keyrings

curl -fsSL https://download.docker.com/linux/ubuntu/gpg | \

sudo gpg --dearmor -o /etc/apt/keyrings/docker.gpg

sudo chmod a+r /etc/apt/keyrings/docker.gpg

# Add Docker repository

echo "deb [arch=$(dpkg --print-architecture) \

signed-by=/etc/apt/keyrings/docker.gpg] \

https://download.docker.com/linux/ubuntu \

$(. /etc/os-release && echo $VERSION_CODENAME) stable" | \

sudo tee /etc/apt/sources.list.d/docker.list > /dev/null

# Install Docker

sudo apt update

sudo apt install -y docker-ce docker-ce-cli containerd.io docker-compose-plugin

## Step 2b: Configure Docker to Run Without sudo

sudo usermod -aG docker $USER

newgrp docker

## Step 2c: Enable Docker Service

# Enable systemd first (edit /etc/wsl.conf)

echo -e '[boot]\nsystemd=true' | sudo tee /etc/wsl.conf

# Restart WSL2 from PowerShell (run this in Windows PowerShell)

# wsl --shutdown

# Then reopen Ubuntu and start Docker

sudo systemctl enable docker

sudo systemctl start docker

## Step 2d: Install NVIDIA Container Toolkit

This allows Docker containers to access your GPU:

curl -fsSL https://nvidia.github.io/libnvidia-container/gpgkey | \

sudo gpg --dearmor -o /usr/share/keyrings/nvidia-container-toolkit-keyring.gpg

curl -s -L https://nvidia.github.io/libnvidia-container/stable/deb/nvidia-container-toolkit.list | \

sed 's#deb https://#deb [signed-by=/usr/share/keyrings/nvidia-container-toolkit-keyring.gpg] https://#g' | \

sudo tee /etc/apt/sources.list.d/nvidia-container-toolkit.list

sudo apt update

sudo apt install -y nvidia-container-toolkit

sudo nvidia-ctk runtime configure --runtime=docker

sudo systemctl restart docker

## Step 2e: Verify Docker + GPU

docker run --rm --gpus all nvidia/cuda:12.4.0-base-ubuntu22.04 nvidia-smi

You should see your RTX 1000 Ada listed inside the Docker container output. If so, everything is working correctly.

TROUBLESHOOTING: If this command hangs or fails with permission errors, run: wsl --shutdown in PowerShell, reopen Ubuntu, then try again.

# Part 3: Set Up Display for Webots GUI

Webots is a graphical simulator. WSL2 supports GUI applications through WSLg (built into Windows 11), but you need to configure the display variable so Docker containers can use it.

## Step 3a: Verify WSLg is Working

Inside Ubuntu, run:

echo $DISPLAY

# Should output something like: :0 or localhost:0.0

If it shows a value, WSLg is working and you can display Linux GUI apps on your Windows desktop.

## Step 3b: Allow Docker to Use the Display

# Allow local connections to X display

xhost +local:docker

# Add this to your ~/.bashrc so it runs automatically

echo 'xhost +local:docker 2>/dev/null' >> ~/.bashrc

## Step 3c: Install X11 Utilities

sudo apt install -y x11-xserver-utils

# Part 4: Install the Booster K1 Webots Simulator

Now that WSL2, Docker, NVIDIA support, and the display are configured, you can pull and run the K1 simulation environment.

## Step 4a: Clone the Webots Simulation Repository

cd ~

git clone https://github.com/socrob/booster_webots_sim.git

cd booster_webots_sim

## Step 4b: Download the Simulation Binaries

The large Webots world files and control runners are distributed separately via GitHub Releases. Run the setup script to download them:

# Make the setup script executable

chmod +x setup.sh

# Run setup (downloads ~2GB of simulation files)

./setup.sh

This will take several minutes depending on your internet connection.

IF SETUP.SH IS MISSING: If setup.sh is not present, check the GitHub Releases page at github.com/socrob/booster_webots_sim/releases for the latest download instructions, as the project is actively maintained.

## Step 4c: Build the Docker Image

make compose-build

This builds the Docker image with Webots, ROS2 Humble, and the Booster SDK pre-installed. First build takes 10-20 minutes.

## Step 4d: Launch the Simulator

# Start the full simulation stack

make compose-up

# Open a shell inside the running container

make exec

The Webots 3D window should appear on your Windows desktop showing the K1 robot in a simulated environment.

## Step 4e: Start the Simulated Robot

Inside the container shell (after make exec):

# Start the K1 simulation world

start-webots

# In a separate terminal (run make exec again for a second shell)

# Start the control runner

start-runner

TIP FOR TESTING: Use start-webots-7dof and start-runner-7dof for the 7 degree-of-freedom simplified model, which is faster and easier to test basic movements.

# Part 5: Connect Your Pipeline Script to the Simulator

This is the payoff. Your k1_voice_pipeline.py connects to the simulator the same way it connects to the real robot — you only change one line.

## Step 5a: Find the Simulator's IP Address

Inside the container shell, run:

hostname -I

Note the first IP address (typically 172.17.0.X or similar). This is your simulator IP.

## Step 5b: Update Your Pipeline Script

In k1_voice_pipeline.py on your laptop, change the K1_IP line:

# Real robot:

K1_IP = "192.168.1.XXX"

# Simulator (replace with your container's IP):

K1_IP = "172.17.0.2"

That is the only change needed. Everything else — Whisper, Ollama, Piper, movement commands — works identically.

## Step 5c: Run the SDK Test First

Before running the full pipeline, verify the SDK can reach the simulator:

# Inside the container, run the SDK example

sdk-client

If the simulated K1 moves in the Webots window, the SDK connection is working.

## Step 5d: Run Your Full Pipeline

From your Windows laptop, open a new Ubuntu terminal and run:

# Make sure Ollama is running first

ollama serve &

# Run the pipeline pointing at the simulator

python3 k1_voice_pipeline.py

Speak into your microphone — the simulated K1 in the Webots window should respond and move.

SIMULATOR LIMITATION: Audio capture from the K1's mic array is not available in the simulator — the AudioCaptureStream will fall back to your laptop's microphone automatically. All movement and speaker output work in simulation.

# Part 6: Daily Startup and Shutdown

Once everything is installed, starting the simulator each day only takes a few minutes.

## Startup Sequence

DAILY STARTUP

| 1 | Open Ubuntu from Start menu (WSL2 terminal) |
| --- | --- |
| 2 | Start Docker: sudo systemctl start docker |
| 3 | Navigate to sim folder: cd ~/booster_webots_sim |
| 4 | Launch containers: make compose-up |
| 5 | Open a container shell: make exec |
| 6 | Inside container, start Webots: start-webots |
| 7 | Open a second container shell: make exec (new terminal) |
| 8 | Inside second shell, start control runner: start-runner |
| 9 | Webots window appears on Windows desktop showing K1 |
| 10 | Run your pipeline: python3 k1_voice_pipeline.py |

## Shutdown Sequence

SHUTDOWN

| 1 | Press Ctrl+C in the pipeline terminal |
| --- | --- |
| 2 | Inside container: stop-webots (or just close the Webots window) |
| 3 | From Ubuntu: make compose-down |
| 4 | Optional: wsl --shutdown in PowerShell to fully stop WSL2 |

# Troubleshooting

## Webots Window Does Not Appear

- Run: echo $DISPLAY  — must show a value like :0

- Run: xhost +local:docker  then try again

- Make sure you are running Windows 11 build 22000 or later (WSLg requires this)

- Try: sudo apt install -y libgl1-mesa-glx  inside the container

## Docker GPU Error: nvidia-container-cli initialization error

- Run: wsl --shutdown in PowerShell, reopen Ubuntu, run: sudo systemctl restart docker

- Verify your Windows NVIDIA driver is version 530+: run nvidia-smi in PowerShell

- Do NOT install any Linux NVIDIA driver inside WSL2

## make compose-build Fails

- Check internet connection — the build downloads several GB of packages

- Run: sudo systemctl status docker  to confirm Docker is running

- Check disk space: df -h  — need at least 20 GB free in WSL2

## SDK Cannot Connect to Simulator

- Confirm start-runner is running in a second container shell

- Verify the container IP with: hostname -I  inside the container

- Update K1_IP in your script to match the container IP

- Check FastDDS profile is loaded: echo $FASTRTPS_DEFAULT_PROFILES_FILE  (should not be empty)

## WSL2 Runs Out of Memory Mid-Simulation

- Increase memory in .wslconfig (see Part 1 Step 1c) — try 16GB if available

- Close other Windows applications before launching Webots

- Use the 7DOF simplified model: start-webots-7dof  instead of start-webots

# Replication at Partner Institutions

This setup was designed to be replicable at any college. The entire simulator runs on a standard Windows 11 laptop with no specialized infrastructure required.

## Minimum Requirements Per Site

- Windows 11 laptop (build 22000 or later)

- NVIDIA GPU — any RTX 2000 series or later with 6+ GB VRAM

- 16 GB RAM (12 GB minimum allocated to WSL2)

- 40 GB free disk space

- Internet connection for initial setup (not required after)

| Setup Phase | Time Estimate |
| --- | --- |
| Part 1: WSL2 + Ubuntu | 15-20 min (includes restart) |
| Part 2: Docker + NVIDIA toolkit | 20-30 min |
| Part 3: Display configuration | 5 min |
| Part 4: Webots K1 container build | 30-60 min (download + build) |
| Part 5: Pipeline connection test | 15 min |
| Total first-time setup | ~2 hours |

TIME-SAVING TIP FOR REPLICATION: After initial setup, a compressed backup of the WSL2 environment can be exported with: wsl --export Ubuntu ubuntu-backup.tar This can be imported on other laptops to skip the build step entirely: wsl --import Ubuntu C:\WSL\Ubuntu ubuntu-backup.tar

# Quick Reference Card

Print and keep at the instructor workstation.

KEY COMMANDS

| What You Want to Do | Command | Where to Run |
| --- | --- | --- |
| Start Docker | sudo systemctl start docker | Ubuntu terminal |
| Launch simulator stack | make compose-up | Ubuntu: ~/booster_webots_sim |
| Open container shell | make exec | Ubuntu: ~/booster_webots_sim |
| Start Webots (full) | start-webots | Inside container |
| Start Webots (simplified) | start-webots-7dof | Inside container |
| Start control runner | start-runner | Inside container (2nd shell) |
| Check container IP | hostname -I | Inside container |
| Stop simulator | make compose-down | Ubuntu: ~/booster_webots_sim |
| Fully stop WSL2 | wsl --shutdown | Windows PowerShell |
| Export WSL2 backup | wsl --export Ubuntu backup.tar | Windows PowerShell |

Hillsborough College — AI Innovation Center — AI PREP4WORK Initiative

This document may be shared freely with partner institutions.